EXPERIMENT -1 :CODE

const int GAS_PIN = A0;

const int PIR_PIN = 2;

const int TRIG_PIN = 9;

const int ECHO_PIN = 8;

const int LED_PIN = 11;

const int BUZZER_PIN = 10;

const int GAS_LIMIT = 400;

const float DISTANCE_LIMIT = 50.0;

void setup() {

pinMode(GAS_PIN, INPUT);

pinMode(PIR_PIN, INPUT);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

Serial.begin(9600);

Serial.println("GAS AND MOTION SECURITY SYSTEM");

Serial.println("==============================");

}

void loop() {

// Gas sensor reading

int gasValue = analogRead(GAS_PIN);

// Convert reading to a relative percentage, not actual gas concentration

int gasLevel = map(gasValue, 0, 1023, 0, 100);

// Motion detection

bool motionDetected = digitalRead(PIR_PIN) == HIGH;

// Ultrasonic distance measurement

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

unsigned long duration = pulseIn(ECHO_PIN, HIGH, 30000UL);

bool distanceValid = duration > 0;

float distance = duration * 0.0343 / 2.0;

// Determine system status

bool gasWarning = gasValue >= GAS_LIMIT;

bool nearbyObject = distanceValid && distance <= DISTANCE_LIMIT;

bool systemSafe = !gasWarning && !motionDetected && !nearbyObject;

// Alarm outputs

digitalWrite(LED_PIN, systemSafe ? LOW : HIGH);

digitalWrite(BUZZER_PIN, systemSafe ? LOW : HIGH);

// Print required information

Serial.print("Gas Level: ");

Serial.print(gasLevel);

Serial.print("% (Raw: ");

Serial.print(gasValue);

Serial.println(")");

Serial.print("Motion: ");

Serial.println(motionDetected ? "DETECTED" : "NOT DETECTED");

Serial.print("Distance: ");

if (distanceValid) {

Serial.print(distance, 1);

Serial.println(" cm");

} else {

Serial.println("NO ECHO");

}

Serial.print("System Status: ");

Serial.println(systemSafe ? "SAFE" : "WARNING");

Serial.println("------------------------------");

delay(1000);

}